Instruction manual of URG Viewer

- Connect a Hokuyo laser scanner to your PC in which the USB driver has been installed.

- Open the software URG Viewer

- Click “Connect” in the application window. The sensor’s performance will be visible. Also the scanning time will be automatically showed in the right-up position of the application.

- Data capture settings

The settings can be changed for multiple purposes. Click on the “window” in the tool bar and open Data Capture Settings.

Mode:

When the scanner receives GD command, it only replies once with the data of 1 scan. When the scanner receives MD command, it will continuously reply to this command until receiving a stopping command. When GD commend is selected, URG Viewer continuous send the GD command to the scanner so the scanner replies to every GD command continuously.

For more information of GD command and MD command, please refer to the protocol SCIP2.0.

Parameter:

- Range: the range (steps) can be changed to show a particular angular area.

- Cluster count: “1” means that the distance data of every step is showed on the screen. “3” means the software only require 1 step’s data and ignore the other 2 steps. This is useful when getting the distance and intensity data together but the unable to receive within the desired period (25msec for UTM-30LX, 100msec for URG-04LX)

- Capture times: if only 1 scan is needed out of 10 scans, this parameter should be set as “9”. The scanning time will be 10 times of every single scan. (e.g. it appears 250msec for UTM-30LX)

- Measured data

Click on “window” in the tool bar and open Measured Data.

Click on “Data Update” in the pop-up-window to check the distance or intensity value of 1 scan.

- Intensity mode

When both distance and intensity data are required, please execute the shortcut of URG Viewer (intensity mode)

Intensity data

Distance data